SECTION 14450

VEHICLE LIFTS

PART 1 – GENERAL

  - SCOPE OF WORK:

- This section specifies equipment to be furnished and installed that will provide vertically stacked car parking.  The Mechanical Parking Equipment (MPE) Subcontractor shall provide car parking machines in the locations shown on the Contract drawings.  The MPE Subcontractor shall provide electro-hydraulic power packs and control systems.  The MPE Subcontractor shall provide all necessary hydraulic piping, valves and control systems.

- The MPE Subcontractor shall perform all electrical wiring needed to make the MPE functional except for the following items which are to be provided by others (normally the Electrical Subcontractor):

  - Provide 208 volt, 30 amp, three phase service to each power pack using #10 wire hot legs and one ground (neutral is not required). The service shall terminate in a fusible disconnect loaded with 25 amp time delay fuses.

  - Provide wiring and conduit from the fusible disconnect to the main control box .  MPE Subcontractor to furnish and install all control wiring and make all control wiring hookups.

  - SUBMITTALS

- Product Data:  Submit Manufacturer’s technical data for MPE.

- Installation Drawings:  Submit shop drawings clearly showing the showing the size and location of each machine, main control box, fusible disconnect, user control keypad, and other components.

- Provide documentation showing the machines and the proposed installation meet 1997 UBC requirements for vertical and seismic loads. Show location of seismic anchorage locations.

  - QUALITY ASSURANCE

- Manufacturer Qualifications:

- Minimum of 15 years of successful experience manufacturing parking lifts of type required for this project.

- A minimum of 5000 platforms of the same model (or earlier versions of the same model) installed in other projects.

    - MPE Subcontractor Qualifications:

      - Minimum of 10 years successful experience installing and servicing the same or similar model lifts.

      - Subcontractor must offer annual maintenance and an extended warranty and be able to provide follow up service as needed.

  - WARRANTY

Provide written warranty agreeing to repair or replace defective parking lifts, and repair, restore, or replace defective work within the specified warranty period.

  - Definition of Defective Parking Lift Operation:  Operation or control system failure, performance below manufacturer’s ratings, excessive wear, unusual deterioration or aging materials or finishes, the need for excessive maintenance or abnormal noise or vibration.

  - Warranty Period:  One year after date of Substantial Completion.

  - This warranty shall be in addition to, and not a limitation of, other rights the Owner may have under the Contract Documents.

  - MAINTENANCE DATA

- Maintenance Manuals:  Provide maintenance manual for MPE with operating and maintenance instructions, emergency instructions, electrical schematics, and similar information.

- Service Tools:  Provide to the Owner any proprietary service tools required for lift.

- Initial Maintenance Service:  Beginning at Substantial Completion, provide three months full maintenance service by skilled, competent employees of the MPE Subcontractor.  Repair or replace worn or defective components, lubricate, and adjust as required for proper operation at rated speed and capacity.  Use parts and supplies as used in the manufacture and installation of original equipment.  Perform maintenance, including emergency callback service, during normal working hours.  Response time shall be next business day or sooner.

- Continuing Maintenance:  Provide a continuing maintenance proposal in the form of an annual maintenance agreement starting on date construction contract maintenance requirements is concluded.  State services provided, obligations, conditions, terms and renewal options.

PART 2 – PRODUCTS

  - MANUFACTURERS

Acceptable Products:  Machines shall be manufactured in accordance with European standard EN 14010. Subject to compliance with requirements, provide Klaus MultiBase complete and ready to operate. Klaus contact information is (925) 284-2092, KlausUSA.com.

  - FABRICATION

- General:  Provide Manufacturer’s standard parking system construction for type and capacity of units indicated.

- Framing shall consist hot-dip galvanized structural and formed steel shapes.

- All bolts, nuts and washers shall be galvanized.

- Platform shall be fabricated from electro-galvanized steel.

- Design and fabricate assembly such that no permanent deformation occurs during normal use of the equipment.  Structural design shall conform to the requirements of the 1997 Uniform Building Code.

- Operation:  Retrieving of cars shall be performed using a magnetic coded key.  Provide 2 for each platform.  Provide electric motor and/or hydraulic pump assembly of proper size, type, and operation for capacity of lifts indicated.

  - Power Units:  Provide Manufacturer’s power units for systems located as shown on the approved shop drawings.

  - Storage & Retrieval Speed:  Shall perform as defined as per approved shop drawings.

  - Wiring:  Coordinate wiring requirements and current characteristics with building electrical system.

    - Rated Capacity:  System platform capacity as defined as per approved shop drawings.

    - Anchorage Provisions:  Provide anchors and fasteners for installation.  Provide types and capacities recommended by Manufacturer for support of loads and conditions of service and exposure.  Anchorages shall conform to the structural requirements of the 1997 Uniform Building Code for vertical and seismic loads.

PART 3 – EXECUTION

  - PREPARATION

Examine conditions under which parking lifts are to be installed and advise Owner if conditions are unsatisfactory for the installation of the MPE before proceeding with installation.

  - INSTALLATION

- General:  Comply with Manufacturer’s detailed instructions in installing parking lifts.

- Hydraulic Tubing Electric Conduit:  Install surface mounted tubing and conduit above the floor and locate away from sources of potential damage.

- Lubrication:  Lubricate operating parts of systems as recommended by Manufacturer.

- Alignment:  Install parking lifts level and plumb accurately in positions indicated.

  - ADJUSTMENT, CLEANING, FINISHING AND PROTECTION

- Make necessary adjustments for safe and efficient operation.

- Clean exposed and semi-exposed surfaces.  Touch-up shop-applied finishes to restore damaged or soiled areas.

- Instruct Owner’s personnel in proper use, operation and maintenance of MPE.  Review emergency provisions and trouble shooting procedures to be followed at time of failure in operation.  Provide one training session for Owner’s personnel.  Provide 2 keys per platform.  Provide 2 copies of maintenance manual for entire project.

2256 Klaus Stacker SpecsPseries.doc